System Architecture & Prompt Design Specification: Local-First DAQ & CA-Ready Server

本文件整合單發（Single-shot）數據擷取系統之架構設計規範、軟體設計模式、DAQ 運作流程與 Antigravity 提示詞，作為建立 Python 儀器控制與 Channel Access (EPICS) Server 的標準 Blueprint。

1. 系統核心設計原則 (Core Architecture Principles)

1.1 在地化優先架構 (Local-First Philosophy)

- 硬體調參與控制在地化：平移台微調定位、相機曝光/Gain/ROI、DG645 延遲時間等複雜設定，均於本地驅動與本地 GUI 完成。

- 本地資料落盤與分析：影像資料直接存入本地 NVMe/SSD 磁碟（HDF5 格式），並由本地程式進行基礎影像分析與 Excel 追加紀錄（experiment_log.xlsx），避免大頻寬影像經由 Channel Access 網路傳輸導致 event loop 阻塞。

1.2 Channel Access (CA) 薄外殼設計 (Thin-Wrapper Pattern)

- 核心邏輯完全封裝於本地 SequenceController 與 DataManager 類別中。

- EPICS CA Server 僅作為輕量化遠端介面，暴露 5 個核心 PV（ShotTarget, Arm, State, FileName, Note），透過呼叫本地 API 觸發流程。

2. 必備軟體設計模式 (Mandatory Software Design Patterns)

為避免生成線性腳本（Procedural Script）導致邏輯混雜與鎖死，程式開發必須嚴格遵守以下四項 OOP 設計模式：

- 策略模式 / 抽象介面 (Strategy Pattern for Drivers)

  - 所有硬體驅動必須繼承抽象基類（BaseCamera, BaseStage, BaseDelayGen），並同時實作實體驅動（RealDriver）與模擬驅動（MockDriver），確保離線狀態下亦可進行完整流程測試。

- 狀態模式與安全鎖 (State Pattern & Interlocks)

  - 系統狀態使用 Enum 強型別管理（IDLE, ARMED, ACQUIRING, SAVING, ERROR）。

  - 執行 Arming 前必須強制校驗安全聯鎖：若馬達靜止狀態 Stage:IsMoving == True，則拒絕待命並拋出 Warning。

- 管理員模式 (Manager Pattern for Data & RAM)

  - 由獨立的 DataManager 類別統一管理 Zero-Copy RAM 記憶體預分配、本地 HDF5 序列化寫入、基礎影像分析與 Excel 追加日誌（experiment_log.xlsx）。

- 異步 Thread 隔離 (Async Thread Pool Isolation)

  - 任何 Blocking I/O（串口通訊、C SDK 呼叫、磁碟寫入）必須透過 asyncio.to_thread() 或 ThreadPoolExecutor 包裝，防止卡死未來的 caproto 事件迴圈。

3. 專案目錄結構 (Project Directory Structure)

Plaintext

DAQ_CA/

├── config.yaml                # 設備 COM Port、波特率、儲存路徑等統一設定檔

├── drivers/                   # 獨立硬體驅動層 (OOP Strategy Pattern)

│   ├── base_driver.py         # 抽象基類 Interface

│   ├── dg645_driver.py        # SRS DG645 驅動 (含 Mock)

│   ├── stage_driver.py        # Faulhaber/Arduino 平移台驅動 (含 Mock)

│   └── camera_driver.py       # Andor & 相機驅動 (含 Mock)

├── core/                      # 本地核心邏輯與資料處理層

│   ├── data_manager.py        # RAM Buffer, HDF5, Excel Logger, 本地分析

│   └── sequence_controller.py # 狀態機主控制器 (預留 CA Hook)

├── test_single_shot_sequence.py # 本地純 Python 整合測試腳本 (Step 2)

└── ca_server/                 # (Phase 2) EPICS IOC 封裝目錄

└── ioc_server.py          # caproto.asyncio 薄外殼服務

4. DAQ 完整運作流程 (DAQ Execution Flow)

Code snippet

flowchart TD

subgraph Step1 ["1. 本地獨立硬體設定 (Local Setup)"]

A[平移台移動至指定位置: Stage:PosVal] --> B[確認平移台完全靜止: Stage:IsMoving == 0]

C[本地設定相機 Exposure / Gain / ROI]

D[本地設定 DG645 脈衝延遲時間]

end

subgraph Step2 ["2. 實驗參數配置 (Experiment Config)"]

E[設定檔名前綴: FileName]

F[設定實驗備註: Note]

G[設定單發/多發發數: ShotTarget = 1 或 10]

end

subgraph Step3 ["3. 待命預備與校驗 (Arming Phase)"]

H[呼叫 controller.arm_and_start] --> I{安全鎖檢查: Stage:IsMoving == 0?}

I -- 移動中 --> J[拒絕 Arming 並拋出 Warning]

I -- 完全靜止 --> K[預先配置 RAM Buffer]

K --> L[切換相機至 External Hardware TTL Trigger 模式]

L --> M[更新系統狀態: State = ARMED]

end

subgraph Step4 ["4. 自動觸發與高速擷取 (Auto-Fire & Capture)"]

M --> N[自動命令 DG645 發射 N 發 TTL 脈衝]

N --> O[相機將高速 TTL 畫面直接寫入預分配 RAM Buffer]

O --> P[即時更新完成進度: ShotCurrent]

end

subgraph Step5 ["5. 本地落盤、分析與自動紀錄 (Persistence & Log)"]

P --> Q[寫入本地 HDF5: exp_run_001.h5]

Q --> R[執行本地基礎圖像分析]

R --> S[自動追加一列紀錄至 central experiment_log.xlsx]

S --> T[復位至 IDLE & 相機解除 Arming]

end

5. 核心 API 實作規範 (sequence_controller.py)

Python

from enum import Enum

import time

from typing import Optional

class SystemState(Enum):

IDLE = "IDLE"

ARMED = "ARMED"

ACQUIRING = "ACQUIRING"

SAVING = "SAVING"

ERROR = "ERROR"

class SequenceController:

def __init__(self, stage_driver, camera_driver, dg645_driver, data_manager):

self.stage = stage_driver

self.camera = camera_driver

self.dg645 = dg645_driver

self.data_mgr = data_manager

self.state = SystemState.IDLE

self.shot_target = 10

self.shot_current = 0

self.file_name = "exp_run"

self.note = ""

def set_experiment_config(

self, file_name: str, note: str, shot_target: int

):

"""設定實驗參數"""

self.file_name = file_name

self.note = note

self.shot_target = shot_target

def arm_and_start(self) -> bool:

"""核心觸發流程：Arming -> Auto-Fire -> Persistence & Analysis

Phase 2 供 EPICS PV (EXP:Seq:Arm = 1) 寫入時直接呼叫

"""

# 1. 安全檢查：平移台是否靜止

if self.stage.is_moving():

print("[Warning] Arming rejected: Stage is still in motion.")

return False

try:

# 2. 進入 ARMED，預分配 RAM 並切換硬體觸發模式

self.state = SystemState.ARMED

self.data_mgr.allocate_ram_buffer(self.shot_target)

self.camera.set_trigger_mode("EXTERNAL_TTL")

# 3. 自動發射 DG645 觸發脈衝

self.state = SystemState.ACQUIRING

self.dg645.set_burst_count(self.shot_target)

self.dg645.trigger()  # 發射 *TRG

# 4. 資料存檔、分析與 Excel 紀錄

self.state = SystemState.SAVING

current_pos = self.stage.get_position()

hdf5_path = self.data_mgr.save_to_hdf5(

file_name=self.file_name, note=self.note, stage_pos=current_pos

)

self.data_mgr.run_local_analysis()

self.data_mgr.append_excel_log(

filename=hdf5_path,

stage_pos=current_pos,

shot_count=self.shot_target,

note=self.note,

)

# 5. 系統復位

self.camera.set_trigger_mode("INTERNAL/OFF")

self.state = SystemState.IDLE

return True

except Exception as e:

self.state = SystemState.ERROR

print(f"[Error] Sequence failed: {e}")

self.camera.set_trigger_mode("INTERNAL/OFF")

return False

6. Phase 2 預留 EPICS PV 規格對照表

| EPICS PV 名稱 | 資料型態 | 讀寫權限 | 對應 SequenceController 方法 / 屬性 |
| --- | --- | --- | --- |
| EXP:Seq:State | ENUM | R | controller.state.value |
| EXP:Seq:Arm | BOOL | W | 寫入 1 時觸發 controller.arm_and_start() |
| EXP:Seq:ShotTarget | INT | R/W | controller.shot_target ($1$ 或 $10$) |
| EXP:Seq:ShotCurrent | INT | R | controller.shot_current ($0 \dots N$) |
| EXP:Data:FileName | STRING | R/W | controller.file_name |
| EXP:Data:Note | STRING | R/W | controller.note |
| EXP:Stage:PosVal | FLOAT | R | stage.get_position() (僅供遠端監控確認) |
| EXP:Stage:IsMoving | BOOL | R | stage.is_moving() |

7. 給 Antigravity 的最終 Prompt 指令

Plaintext

TASK: Implement Local-First Single-Shot DAQ Framework (Phase 1 & Phase 2 Ready)

Please implement the local DAQ control framework following the architecture specified in this document:

1. Software Design & OOP Requirements:

- Implement `drivers/` using Strategy Pattern (Abstract Base Classes with both Real and Mock implementations for DG645, Stage, and Cameras).

- Implement `core/data_manager.py` using Manager Pattern to handle zero-copy RAM pre-allocation (`numpy`), local HDF5 persistence (`h5py`), basic image processing, and auto-appending summary records to `experiment_log.xlsx` (`openpyxl`/`pandas`).

- Implement `core/sequence_controller.py` using State Pattern (`SystemState` Enum: IDLE, ARMED, ACQUIRING, SAVING, ERROR).

2. Interlock & Execution Constraints:

- `arm_and_start()` must strictly enforce the interlock: if `stage.is_moving() == True`, abort arming and return False.

- Stage positioning and camera exposure tuning are handled independently on the local PC prior to calling `arm_and_start()`.

- Upon `arm_and_start()`, pre-allocate RAM, switch cameras to EXTERNAL_TTL trigger mode, fire DG645 burst automatically, persist data to local HDF5, run analysis, append Excel log, and return to IDLE.

3. Execution Steps:

- Step 1: Implement clean driver interfaces and mock drivers in `drivers/`.

- Step 2: Implement `data_manager.py` and `sequence_controller.py`.

- Step 3: Deliver `test_single_shot_sequence.py` to validate both N=1 and N=10 acquisition runs locally in pure Python without EPICS.

- Keep all APIs clean and isolated so `caproto.asyncio` PV callbacks can directly bind to `sequence_controller.py` in Phase 2.