System Architecture & Prompt Design Specification: Local-First DAQ & CA-Ready Server

This document serves as the updated technical design blueprint, architecture specification, and master context prompt for Antigravity to build low-level Python hardware drivers, the dual GigE camera GUI, the caproto Channel Access (EPICS) server, and the project logbook.

1. Core Architecture Principles

1.1 Local-First Philosophy

- Local Control & Tuning: Stage micro-positioning, camera exposure/gain/ROI, and DG645 delay channels are configured and operated locally.

- Local Data Persistence & Processing: Images are stored directly to local SSD in HDF5/TIFF formats with metadata tags. Local image processing (e.g., 2D Gaussian fitting) and automatic Excel log appending (experiment_log.xlsx) run locally to avoid EPICS event loop network bottlenecks.

1.2 EPICS Channel Access Thin-Wrapper Pattern

- All business logic is encapsulated inside local SequenceController and DataManager modules.

- The caproto IOC acts strictly as a lightweight remote wrapper, exposing 5 primary PVs (ShotTarget, Arm, State, FileName, Note).

2. Mandatory Software Design Patterns

To prevent procedural spaghetti code and event loop blocking, code generation must strictly adhere to these four Object-Oriented design patterns:

- Strategy Pattern / Abstract Interfaces for Drivers:

  - All drivers inherit from abstract base classes (BaseCamera, BaseStage, BaseDelayGen).

  - Implement both RealDriver and MockDriver for offline development and testing.

- State Pattern & Interlocks:

  - Enforce strict state management via Enum (IDLE, ARMED, ACQUIRING, SAVING, ERROR).

  - Interlock rule: arm_and_start() must verify stage.is_moving() == False before arming cameras.

- Manager Pattern for Data & RAM:

  - DataManager handles zero-copy RAM buffer pre-allocation, local HDF5 persistence, TIFF tag embedding, and Excel log appending (experiment_log.xlsx).

- Thread Pool Isolation:

  - All blocking I/O (Serial RS-232, C SDK calls, file saving) must be wrapped using asyncio.to_thread() or ThreadPoolExecutor.

3. Project Directory Structure

Plaintext

DAQ_CA/

├── DEVELOPMENT_LOG.md         # Token-efficient logbook tracking APIs, state & tasks

├── config.yaml                # Serial ports, baud rates, default paths

├── drivers/                   # Hardware Driver Layer (Strategy Pattern)

│   ├── base_driver.py         # Abstract base classes

│   ├── dg645_driver.py        # SRS DG645 driver (with Mock)

│   ├── stage_driver.py        # Translation stage driver (with Mock)

│   └── camera_gui.py          # Dual GigE Camera GUI & Driver (with Mock)

├── core/                      # Local Core Logic & Data Layer

│   ├── data_manager.py        # RAM Buffer, HDF5, TIFF Tags, Excel Logger

│   └── sequence_controller.py # State machine controller (CA Hook)

├── test_single_shot_sequence.py # Pure-Python local integration test script

└── ca_server/                 # EPICS IOC Directory (Phase 2)

└── ioc_server.py          # caproto.asyncio thin-wrapper server

4. Logbook & Context Maintenance Rules (DEVELOPMENT_LOG.md)

To optimize token usage and prevent context drift across development sessions, DEVELOPMENT_LOG.md must be maintained at the project root with the following structure:

- System Architecture Summary: Concise recap of design patterns, local-first rules, and interlocks.

- Completed Modules & API Contracts: Signatures of implemented classes/methods (e.g., SequenceController.arm_and_start()).

- Current Task / Next Milestone: Clear specification of the active task.

- Known Technical Notes: Specific implementation decisions (e.g., 10 Hz GUI throttle, raw background TIFF tags).

Rule for Antigravity: Read DEVELOPMENT_LOG.md at the start of every session to establish context, and update DEVELOPMENT_LOG.md upon completing or revising any module.

5. DAQ Execution Flow

Code snippet

flowchart TD

subgraph Step1 ["1. Local Independent Setup"]

A[Move Stage to Target Position: Stage:PosVal] --> B[Confirm Stage Static: Stage:IsMoving == 0]

C[Configure Camera Exposure / Gain / ROI Locally]

D[Set DG645 Pulse Delays Locally]

end

subgraph Step2 ["2. Experiment Configuration"]

E[Set Filename Prefix: FileName]

F[Set User Notes: Note]

G[Set Shot Count: ShotTarget = 1 or 10]

end

subgraph Step3 ["3. Arming & Interlock Check"]

H[Call controller.arm_and_start] --> I{Interlock: Stage:IsMoving == 0?}

I -- Moving --> J[Reject Arming & Return Warning]

I -- Static --> K[Pre-allocate RAM Buffer]

K --> L[Switch Cameras to EXTERNAL_TTL Mode]

L --> M[Set State = ARMED]

end

subgraph Step4 ["4. Auto-Fire & High-Speed Capture"]

M --> N[Automatically Issue DG645 Burst: N Pulses]

N --> O[Cameras Capture TTL Frames directly into Pre-allocated RAM]

O --> P[Update Progress: ShotCurrent]

end

subgraph Step5 ["5. Local Persistence, Analysis & Logging"]

P --> Q[Save Raw Data to Local HDF5 / TIFF with Tags]

Q --> R[Execute Local 2D Gaussian Fit Analysis]

R --> S[Append Row to Central experiment_log.xlsx]

S --> T[Reset State to IDLE & Disarm Cameras]

end

6. Target EPICS PV Mapping

| EPICS PV Name | Type | Access | Corresponding Method / Attribute |
| --- | --- | --- | --- |
| EXP:Seq:State | ENUM | R | controller.state.value |
| EXP:Seq:Arm | BOOL | W | 1 triggers controller.arm_and_start() |
| EXP:Seq:ShotTarget | INT | R/W | controller.shot_target ($1$ or $10$) |
| EXP:Seq:ShotCurrent | INT | R | controller.shot_current ($0 \dots N$) |
| EXP:Data:FileName | STRING | R/W | controller.file_name |
| EXP:Data:Note | STRING | R/W | controller.note |
| EXP:Stage:PosVal | FLOAT | R | stage.get_position() |
| EXP:Stage:IsMoving | BOOL | R | stage.is_moving() |

7. Master Antigravity Prompt

Plaintext

TASK: Implement Local-First Single-Shot DAQ & Camera Framework with DEVELOPMENT_LOG.md

Please implement the system according to the specifications in this document:

1. Context & Logbook Management:

- Create and maintain `DEVELOPMENT_LOG.md` at the root directory.

- Read `DEVELOPMENT_LOG.md` first before generating new code to maintain token efficiency and architecture context.

- Update `DEVELOPMENT_LOG.md` whenever a module, driver, or API signature is finalized or updated.

2. Software Design & OOP Requirements:

- Drivers (`drivers/`): Use Strategy Pattern with Abstract Base Classes (`base_driver.py`) and provide both Real and Mock implementations for DG645, Stage, and Dual GigE Cameras.

- Dual Camera Module (`camera_gui.py`): Support 2x GigE cameras with user-named channel textboxes, 10 Hz display throttle, raw live view, beam overlay tool, 2D Gaussian fitting over ROI, background capture, and TIFF tag metadata embedding.

- Data Manager (`core/data_manager.py`): Pre-allocate zero-copy RAM buffers (`numpy`), handle HDF5/TIFF persistence, and auto-append run summary rows to `experiment_log.xlsx`.

- Controller (`core/sequence_controller.py`): Use State Pattern (`SystemState` Enum). Enforce strict interlock checking (`stage.is_moving() == False`).

3. Execution Order:

- Step 1: Create `DEVELOPMENT_LOG.md` and implement `drivers/` (including Mock drivers).

- Step 2: Implement `core/data_manager.py` and `core/sequence_controller.py`.

- Step 3: Implement `test_single_shot_sequence.py` to validate 1-shot and 10-shot runs locally without EPICS.

- Step 4: Ensure all APIs are clean and thread-isolated (`asyncio.to_thread()`) for caproto EPICS IOC integration in Phase 2.