System Architecture & Prompt Design Specification: Python Channel Access Server

This document serves as the technical design blueprint and context prompt for Antigravity to build low-level Python hardware drivers, the caproto Channel Access (EPICS) server, and the user interface for the ultrafast laser single-shot acquisition system.

1. 3-Phase Development Roadmap

┌─────────────────────────────────────────────────────────┐

│ Phase 3: User Interface / Client (PyQt / PyDM)          │

└───────────────────────────┬─────────────────────────────┘

│ EPICS Channel Access (PVs)

┌───────────────────────────▼─────────────────────────────┐

│ Phase 2: caproto IOC Server (Asynchronous PV Callbacks)  │

└───────────────────────────┬─────────────────────────────┘

│ Python API Method Calls

┌───────────────────────────▼─────────────────────────────┐

│ Phase 1: Standalone Hardware Drivers & Unit Tests       │

└──────────┬───────────────┬─────────────────┬────────────┘

│ Serial        │ Serial          │ C/C++ SDK

┌─────▼─────┐   ┌─────▼─────┐     ┌─────▼─────┐

│   DG645   │   │  Arduino  │     │   Andor   │

│  (RS-232) │   │  (Stage)  │     │  Camera   │

└───────────┘   └───────────┘     └───────────┘

2. Phase 1: Low-Level Hardware Drivers & Specifications

A. Digital Delay Generator: SRS DG645 (dg645_driver.py)

- Communication Interface: RS-232 via pyserial (baud rate 19200, 8N1).

- Protocol: ASCII SCPI commands.

- Key Commands:

  - BURS:NCYC 10: Set burst count to 10 cycles.

  - TSRC 0: Set trigger source to Internal/Single.

  - *TRG: Software trigger command.

- Target Python Interface:

class DG645Driver:

def connect(self, port: str, baudrate: int = 19200): ...

def set_burst_count(self, shots: int = 10): ...

def set_delay(self, channel: str, delay_s: float): ...

def trigger(self): ...

B. Translation Stage: Arduino Controller (stage_driver.py)

- Communication Interface: USB-Serial / RS-232 via pyserial.

- Custom ASCII Protocol:

  - MOVE <float>: Command absolute move target in mm (Arduino responds with OK\n).

  - POS?: Query current position (Arduino responds with <float>\n).

  - STOP: Emergency motor stop (Arduino responds with OK\n).

  - STAT?: Query motion status (Arduino responds with IDLE\n or MOVING\n).

- Target Python Interface:

class ArduinoStageDriver:

def move_absolute(self, target_mm: float) -> bool: ...

def get_position(self) -> float: ...

def is_moving(self) -> bool: ...

C. Detectors: Andor & Standard Cameras (camera_driver.py)

- Trigger Mode: External hardware TTL trigger mode.

- Buffering Strategy: Pre-allocate RAM buffer arrays for 10-shot image sequence prior to triggering to eliminate acquisition lag.

3. Master Antigravity Context & Generation Prompt

TASK: Python EPICS Channel Access (CA) Server & Hardware Control

1. System Overview

Build a modular Python system using caproto for EPICS Channel Access server functionality, driving an ultrafast laser single-shot experiment across a DG645 (RS-232), custom Arduino translation stage, and cameras (Andor + standard cameras).

2. Trigger & Handshake Architecture (Hardware-Armed Asynchronous State Machine)

[Master Controller] --(Arm & Set Metadata)--> [caproto EPICS Server]

|

(Pre-arm Devices)

|

[Check System:Ready == 1]

|

(Fire DG645 Trigger Train)

|

[DG645] --(TTL Hardware Triggers)--> [Andor] [Cams] [Stage]

|       |       |

(Hardware Capture & Buffer)

|

(Async HDF5 / Metadata Save)

Handshake State Logic:

- Idle Phase: User configures experiment parameters (Stage target position, Exposure times, File base name, Shot count = 10).

- Arm Phase (Seq:Arm = 1):

  - EPICS Server commands cameras to switch to External Hardware Trigger Mode.

  - Translation stage moves to target position and locks (Stage:IsMoving == 0).

  - Devices set [Device]:Ready = 1.

- Global Ready Check:

  - Server checks aggregate readiness: System:Ready = (Andor:Ready AND Cams:Ready AND Stage:Ready).

- Trigger Phase:

  - Once System:Ready == 1, server enables DG645 outputs or issues software trigger command *TRG to DG645 to output a 10-pulse burst.

- Acquisition & Logging Phase:

  - Detectors capture image data into pre-allocated RAM buffers via TTL lines.

  - Server writes HDF5 dataset asynchronously (Timestamp, Shot ID, Stage Position, Image Frames).

- Done Phase:

  - Server sets Seq:State = IDLE.

4. EPICS PV (Process Variable) Naming Scheme

| PV Name | Type | Access | Description |
| --- | --- | --- | --- |
| EXP:Seq:State | ENUM | R | System state (IDLE, ARMING, READY, ACQUIRING, ERROR) |
| EXP:Seq:Arm | BOOL | W | 1 = Prepare hardware for acquisition sequence |
| EXP:Seq:ShotTarget | INT | W | Target number of shots per condition (default: 10) |
| EXP:Seq:ShotCurrent | INT | R | Current shot index completed |
| EXP:Data:FileName | STRING | W | Base filename / output prefix for current run |
| EXP:Data:MetaLog | STRING | R | JSON formatted string containing shot metadata |
| EXP:DG645:TriggerStart | COMMAND | W | Software trigger command to DG645 |
| EXP:Stage:PosVal | FLOAT | R/W | Target/Current position of translation stage (mm) |
| EXP:Stage:IsMoving | BOOL | R | 0 = Stopped/Ready, 1 = Moving |
| EXP:Andor:AcqStatus | ENUM | R | Camera readiness and buffer status |

5. File & Data Organization

All 10 shots of an experimental condition are saved into a single unified HDF5 file (h5py):

experiment_run_001.h5

├── /metadata (Stage position, Laser parameters, Timestamps)

├── /shot_00

│     ├── andor_image

│     └── cam1_image

├── ...

└── /shot_09

6. Antigravity Coding Workflow Guidelines

- Step 1 (Drivers): Generate standalone pure-Python drivers (dg645_driver.py, stage_driver.py, camera_driver.py) with executable if __name__ == '__main__': test blocks.

- Step 2 (Integration Script): Generate test_single_shot_sequence.py to test a 10-shot sequence directly in Python without EPICS.

- Step 3 (caproto IOC): Wrap proven drivers inside caproto.asyncio PV groups to expose control over Channel Access.

- Step 4 (UI): Build a PyQt / PyDM graphical interface connecting to the Channel Access PVs.